GEOSPATIAL ANALYSIS

The spatial analysis of groundwater quality parameters highlights significant regional variability within the study area. Electrical Conductivity (EC) ranging from 676 µS/cm to 8900 µS/cm, with an average of 2272 µS/cm, EC values are highest in the central and southern regions, indicating elevated ion concentrations and higher salinity. Conversely, lower values in the central and northern regions suggest better water quality in these areas. Total Dissolved Solids (TDS) concentrations range between 473 mg/L and 6230 mg/L, with an average of 1589 mg/L. Elevated TDS levels, observed in the southern and central parts, correlate with poorer water quality, while lower values in the northern areas reflect more favorable water conditions. pH varies from 6.2 to 7.99, with an average of 7.41, indicating neutral to slightly alkaline water. Higher pH values are predominantly located in the western region, suggesting mildly alkaline conditions, while the lower values in some areas signal slightly acidic water. Sodium (Na) levels are concentrated in the central and southern parts, further contributing to salinity concerns. The northern areas exhibit lower sodium concentrations, suggesting a lesser impact from salinity. Calcium (Ca) with a range of 35 mg/L to 640 mg/L, higher calcium concentrations are observed in the central and southern regions, correlating with areas experiencing hard water. Northern areas show lower calcium levels, indicating better water quality in terms of hardness. Magnesium (Mg) varying between 13 mg/L and 192 mg/L, magnesium concentrations mirror the spatial pattern of calcium, with higher levels found in the central and southern regions, further contributing to water hardness. Chloride (Cl) concentrations range from 36 mg/L to 2100 mg/L, with the southern areas exhibiting higher chloride levels, likely driven by evaporation and dissolution processes, contributing to saline water conditions. Elevated sulfates (SO₄²⁻) concentrations are found in the southern region, while bicarbonates (HCO₃⁻) are more prominent in the western parts, likely resulting from carbonate dissolution processes. Higher nitrates (NO₃⁻) levels in the southern part suggest contamination from agricultural runoff or sewage, posing a potential risk to water quality. Total Hardness (TH): Hard water is prevalent in the central and southern parts of the study area, primarily due to elevated concentrations of calcium and magnesium.