Results and discussion

To understand the solvation structure of electrolytes was performed Raman spectroscopy and compared with low concentration to high concentration DME electrolytes. The S-N-S vibrational stretching frequency of FSI- is located in the range of 700-780 cm-1. By deconvoluted the peaks different types of solvation clusters were observed. From 0.1M LiFSI in DME to 3M LiFSI in DME increased aggregation of electrolytes clears that by adding more salt interaction with solvent getting weakened and interaction with FSI- getting stronger with Li+. However, in case of three ether isomers much more aggregation of Li+ with FSI- were detected compare to DME electrolytes.  Suggesting stronger interaction of Li+ with FSI- than high concentration DME electrolyte. The fraction of Aggregations (AGGI and AGGII) increases in the trend of DPE1 (100%AGGII)>BEE1 (27.33%AGGI,72,67%AGGII)>PME1 (69.61%AGGI). Whereas CIP observed only in PME1(30.39%). Which indicates more FSI- is coordinated with Li+ in DPE1 electrolyte and by reduction of FSI- anions more inorganic rich SEI forms compare to BEE1 and PME1.

Again, this result was proved by NMR (7Li and 19F). Where a downfield shift of the 7Li was observed in the trend of DME>PME>BEE>DPE. Which indicates increased FSI- and Li+ coordination and decreased Li+ and solvent coordination follow the same trend. This is because as DME is strongly polar solvent and it has strong ability to coordinate with Li+. Between three positional isomers DPE has relatively low polarity and also two propyl group create steric hindrance, reducing its ability to coordinate effectively with lithium ions compared to PME which has highest polarity between three ethers and as it has one side pentyl group but other side is short methyl group, so it has relatively low steric hindrance and has relatively high coordination power with Li+. Where BEE feels moderate steric hindrance and moderate coordinating ability. As a result, stronger coordination with lithium ions leads to more shielding and thus an upfield shift is observed in the trend of DME>PME>BEE>DPE.

But in case of 2M concentration more upfield shift observed compare to 1M concentraton. This is because of adding more number of salt total electron cloud density on Li + increases. Resulting more upfield shft in higher concentration .

In 19F NMR also observed a downfield shift as solvent changes from DME, PME, BEE, DPE.

Fourier-transform infrared (FTIR) spectroscopy was utilized to analyze the solvation structure of various electrolytes. The peaks at 1385 cm−1 and 1364 cm−1 represent the Li+-FSI− interaction and free FSI−, respectively. As the electrolyte shifted from 2 M LiFSI-DME to 2 M LiFSI-DMP and then to 2 M LiFSI-TFDMP, these peaks widened and diminished in intensity. This indicates greater coordination of Li+-FSI− and less free FSI−.

Fourier-transform infrared (FTIR) spectra also further proved (Supplementary Fig. 43) the unique solvation structure in DTDL based electrolyte. Unlike DME, a strong FSI anion−Li+ coordination peak was detected in both 1 M and 2 M LiFSI-DTDL electrolytes

infrared (FTIR) spectroscopy of pure solvents DME, lithium salt LiFSI, and four kinds of electrolytes to further investigate the intermolecular interactions between DME and LiFSI with different concentrations (shown in Figure S2a). It can be seen that as the electrolyte concentration increased, the characteristic peak of C-O-C of 1130 displayed a red shift little by little. DH(3/5)-1.5M-LiFSI and HCE had similar infrared absorption vibration peaks. And with the lithium salt additive LiCO2CF3, DH(3/5)-1.65M-LiFC not only shifted to a lower wavenumber at 1107 cm−1 but also became broader, reflecting the complexation between the lithium salt and the solvent was stronger. Due to complexation with Li+ of lithium salts, the DME oxygen donates the lone pairs to Li+ , resulting in the lowering of the HOMO energy level of DME 42, which is also consistent with the results of LSV. The scheme representative of the interfacial stability of cathode enabled by the lower HOMO of DME-LiFSI-HFE-LiCO2CF3 and CEI is shown in Figure S2b