FHP = Flexible heat pipe

OD = Outer Diameter

L = Length

T = Thickness

W = Width

D = Diameter

SS = Stainless Steel

Ice and snow melting using geothermy

| Sl. No. | Year | Authors | Type of Wick | Working Fluid | Dimensions | Flexible material | Type of heat pipe | Features |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1993 | Shiraishi et al. | Not disclosed | Not disclosed | D = 2.56cm, L = 400cm | Corrugated flexible element | Long flexible corrugated heat pipe | Proposed a snow melting system using long flexible heat pipe |
| 2 | 2017 | Wang et al. | Metallic membrane | Ammonia | D = 3.2cm, L = 7000cm | thermally conductive polymer | Super-long flexible heat pipes | Suggested a super long flexible heat pipe to keep the pavements free from ice and snow throughout the calendar year |
| 3 | 2017 | Wang et al. | Metallic membrane | Ammonia | D = 3.2cm, L = 7000cm | thermally conductive polymer | Super-long flexible heat pipes | A numerical study was conducted to optimize the operating parameters |

Space heating using geothermy

| Sl. No. | Year | Authors | Type of Wick | Working Fluid | Dimensions | Flexible material | Type of heat pipe | Features |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2018 | Wang et al. | Metallic membrane | Ammonia | D = 3.2cm, L = 7000cm | thermally conductive polymer | Super-long flexible heat pipes | Proposed a hybrid ground source heat pump system employing the super-long flexible heat pipes |
| 2 | 2020 | Liu et al. | Metallic membrane | Ammonia | D = 3.2cm, L = 7000cm | thermally conductive polymer | Super-long flexible heat pipes | conducted an experimental study to assess the thermal performance of a super-long flexible heat pipe |
| 3 | 2021 | Liu et al. | Metallic membrane | Ammonia | D = 3.2cm, L = 7000cm | thermally conductive polymer | Super-long flexible heat pipes | Studied soil temperature fluctuation characteristics using super-long flexible heat pipes |
| 4 | 2023 | Liu et al. | Wickless | Ammonia | D= 5.5cm, L = 5250cm | SS 304 corrugated-pipe | Super-long ﬂexible thermosiphons | Evaluated the thermal performance of innovative geothermal thermosiphons. |

Space radiator applications

| Sl. No. | Year | Authors | Type of Wick | Working Fluid | Dimensions | Flexible material | Type of heat pipe | Features |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1975 | Edelstein F | SS mesh | Ammonia | D=0.7874cm, L=269.24cm | SS bellow | Cylindrical flexible heat pipe | A single fluid flexible heat pipe header to transfer heat to the radiator |
| 2 | 1984 | Merrigan et al. | SS mesh | Sodium | L=80cm | SS bellow | Cylindrical flexible heat pipe | Developed 1800 bendable sodium/SS heat pipe |
| 3 | 1985 | Shaubach et al. | ‘V’ groove, sintered SS powder metal, SS mesh | Acetone | D = 1.6cm, L = 73.7cm | Copper bellows | High performance flexible heat pipes | Created a flexible heat pipe prototype for use in space |
| 4 | 1988 | Hwangbo | SS screen with helical grooves | Ammonia | D = 2.5cm, L = 136cm | SS bellows | Flexible variable conductance heat pipe | Variable conductance heat pipe designed for a deployable radiator |
| 5 | 1990 | Trujillo et al. | SS mesh | Sodium | L = 100cm, W = 8cm | SS foil | High-temperature, deployable, membrane flat heat pipe | A lightweight, flexible, flat heat pipe for deployable radiators was developed. |
| 6 | 1998 | Fale et al. | Wickless | Water | D = 1.9cm, L = 100cm | Flexible laminate material | Flexible heat pipe heat rejection radiator | Fully functional deployable radiator utilizing flexible heat pipe for lunar application |
| 7 | 1999 | Glass et al. | Ultra-high molecular-weight film wick | Acetone, Freon-11, Methanol, and Ethanol | D = 2.54cm, L = 70-80cm | Kapton and aluminized Mylar | Flexible heat pipes utilizing ultra-high-molecular-weight film wicks | Successfully showcased the practicality of producing flexible polymeric heat pipes suitable for utilization in radiators for spacecraft |
| 8 | 2002 | Krotiuk et al. | Porous sintered nickel wick | Propylene | Length of transport line = 733cm | SS braided bellow | Propylene loop heat pipe | Developed a flexible, lightweight, and deployable radiator for space applications |
| 9 | 2006 | Abe et al. | Wickless | 1-Butanol Aqueous Solution, Ethanol Aqueous Solution | D = 0.8cm, L = 25cm | Polyimide film | Flexible Wickless Heat Pipes Radiator | shown the benefit of using self-rewetting fluids in microgravity conditions |
| 10 | 2009 | Tanaka et al. | Wickless | Water, 1-Butanol aqueous solutions, 1-Heptanol aqueous solution | D = 0.8cm, L = 24cm | Polyimide film | Ultralightweight polyimide-based wickless heat pipe | Presented the idea of ultra-lightweight wickless heat pipes with self-rewetting fluids |
| 11 | 2019 | Ambrose | Proprietary Details | Ammonia | Length of transport line = 27cm | EDU Flex Hose | Dual-flex heat pipe | proposed using a dual flex heat pipe to connect the east and west radiators thermally. |

High power electronics and sensor cooling

| Sl. No. | Year | Authors | Type of Wick | Working Fluid | Dimensions | Flexible material | Type of heat pipe | Features |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1975 | Saaski et al. | SS mesh | Methane | D = 2.39cm, L=124.46cm | Braided SS bellow | Cylindrical cryogenic flexible heat pipe | Developed a cryogenic flexible heat pipe which transfers 20W of heat under normal operating temperatures of the IR sensor |
| 2 | 1976 | Wright et al. | Composite SS Mesh | Methane & Oxygen | D = 0.95cm | Braided SS bellow | High performance flexible cryogenic heat pipes | Demonstrated the feasibility of flexible cryogenic heat pipes |
| 3 | 1989 | Nelson J. Gernert | Sintered copper powder metal with braided wire cable artery | Water | L = 74cm, cold plate length =13cm | Copper expanded bellows | Flexible cylindrical heat pipe cold plate | A flexible heat pipe cold plate has been developed to provide thermal protection for the electronics of flight actuators. |
| 4 | 1991 | Nelson Gernert et al. | Sintered copper powder metal with braided wire cable artery | Water | L = 73.66cm (for version 1&2) L = 81.28cm (for version 3&4) | Copper expanded bellows | Flexible cylindrical heat pipe cold plate | Four heat pipe cold plates capable of addressing thermal management challenges in aircraft were developed |
| 5 | 1997 | Thomas et al. | Not disclosed | Water | Not disclosed | Not disclosed | Flexible copper-water heat pipe | Studied the effect of transient acceleration loading on the thermal resistance of flexible copper-water heat pipe |
| 6 | 1998 | Thienel et al. | SS mesh | Oxygen and Methane | L = 0.55m (oxygen) L = 0.65m (methane) | SS double braided bellow | Cryogenic flexible diode heat pipes | Demonstrated the feasibility of cryogenic flexible diode heat pipes under microgravity conditions |
| 7 | 2003 | Hoang et al. | Sintered powdered SS wick | Nitrogen | Length of transport line = 254cm D = 0.238cm | SS tube helical flexure mechanism | Cryogenic advanced Loop Heat Pipe | Cryogenic cooling across gimbal using flexible cryogenic loop heat pipes |
| 8 | 2005 | Hoang et al. | Sintered powdered SS wick | Nitrogen | Length of transport line = 254cm D = 0.238cm | SS tube helical flexure mechanism | Cryogenic advanced Loop Heat Pipe | shown through experimentation the mechanical and thermal capabilities for cryocooling across gimbal joints |
| 9 | 2006 | Thayer et al. | Sintered nickel powder wick | Ammonia | Not disclosed | Coiled aluminium SS tubes | Flexible air-cooled loop heat pipe | Flexible loop heat pipes with thermoelectric coolers to enhance the overall performance |
| 10 | 2018 | Der et al. |  | Ethanol | L = 25cm, W = 9cm | Polypropylene sheet | Flexible pulsating heat pipe | Developed a novel flexible pulsating heat pipe using polypropylene sheets |
| 11 | 2019 | Rathod et al. | SS mesh | Acetone | D = 0.9cm, L = 20cm | SS bellow | Flexible cylindrical heat pipe | SS-Acetone flexible heat pipe was developed and evaluated for thermal performance and mechanical flexibility |
| 12 | 2023 | Clemens et al. |  | FC72, ethanol | L = 26cm, T = 0.15cm | Flat polypropylene sheets | Flexible pulsating heat pipe | Studied the effect of bending and microgravity conditions on performance |

Heat pipe as thermal strap

| Sl. No. | Year | Authors | Type of Wick | Working Fluid | Dimensions | Flexible material | Type of heat pipe | Features |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2017 | Wilson et al. |  | R-134a | L = 27cm, W = 6cm | Polymeric tubing | Flexible oscillating heat pipe | Suggested an enhanced thermal strap employing flexible oscillating heat pipe which can act as active heat transfer device |
| 2 | 2021 | Iwata et al. |  | HFC-134a | L = 18.3cm, W = 4cm | SUS304 capillary tube | Micro-oscillating heat pipe | A flexible thermal strap was developed using a micro-oscillating heat pipe, and its dynamic stiffness was evaluated |
| 3 | 2022 | Nagano et al. |  | Pure water | L = 31.5cm, T = 0.06cm | Six layers of pure copper foils | Flexible ultrathin loop heat pipe | Developed a flexible ultra-thin loop heat pipe and its performance was evaluated before and after freezing |

Cooling of power electronics

| Sl. No. | Year | Authors | Type of Wick | Working Fluid | Dimensions | Flexible material | Type of heat pipe | Features |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2003 | Kobayashi et al. | 1st Wick-Teflon 2nd wick polyethylene And groove | R134a | Total length of circulation line = 750cm | Flexible transporting tube | Flexible Looped Heat Pipe | Flexible loop heat pipe with long transport lines and high capacity was developed |
| 2 | 2011 | Wu et al. | Copper mesh | Methanol | L = 7.5cm, W = 2cm | Polyethylene-terephthalate | Flexible micro heat pipe | Low cost flexible lightweight micro heat pipe with high effective thermal conductivity |
| 3 | 2014 | Vasiliev et al. | Rectangular capillary grooves on evaporator and condenser | R 600 | Transport line length = 20cm | Flexible vapour and liquid transport pipes made from pure polyamide | Flat horizontal polymer loop thermosyphon | Polymer based composite material thermosiphon with high thermal conductivity |
| 4 | 2016 | Shi et al. | Copper mesh | Deionized water and acetone | L = 8cm, W = 5cm | Flexible polymer | Polymer-based ﬂat heat pipe | Thermal resistance was less than copper equivalent heat pipe |
| 5 | 2020 | Dai et al. | Copper mesh | Deionized water | L = 8cm, W = 0.9cm | Flexible copper | Flexible ultra-thin flattened heat pipes | Incorporated flexibility in ultra-thin flat heat pipes and studied the effect of bending |
| 6 | 2021 | Udaybhanu et al. | Not disclosed | Deionized water | Not disclosed | Not disclosed | Flexible cylindrical heat pipe | Numerically analysed the thermal performance at different bending angles |
| 7 | 2023 | Yang et al. | Spiral woven mesh adhered to a coarse copper mesh | Deionized water | L = 14cm, W = 1.4cm | Flexible copper foil | Composite flexible ﬂat heat pipe | Developed a high performance flexible flat heat pipe |
| 8 | 2023 | Han et al. |  | Ethanol | L = 8.7cm, W = 2.5cm | thermoplastic polyurethane elastomer | Polymer-based flexible oscillating heat pipe | A low cost flexible oscillating heat pipe with high effectiveness |

Cooling of foldable and wearable electronics

| Sl. No. | Year | Authors | Type of Wick | Working Fluid | Dimensions | Flexible material | Type of heat pipe | Features |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2007 | Miyazaki |  | R-134a | Wing type, D = 0.1 to 0.17cm Straight type, D = 0.1cm | Teflon flexible tubes | Flexible oscillating heat pipes | Proposed the idea of using the back side of the display of notebook/PC as radiating surface by employing an oscillating heat pipe |
| 2 | 2011 | Oshman et al. | Hybrid copper woven mesh | Water | L = 6cm, W = 3cm | Liquid-crystal polymer films | Polymer-based flat heat pipes | Polymer based non-electrically conductive flat heat pipe |
| 3 | 2013 | Hsieh et al. | Copper mesh | Deionized water | L = 8cm, W = 2cm | Silicone rubber | Polymer-based flexible flat heat pipe | Polymer based flexible heat pipe having mesh type pillar |
| 4 | 2015 | Chao Yang | Superhydrophil-ic strong-base-oxidized copper mesh | Deionized water | L = 40cm, D = 0.5cm | Flexible polyurethane polymer connector | Flexible cylindrical heat pipe | Bioinspired high performance flexible heat pipe |
| 5 | 2016 | Chao Yang | Superhydrophil-ic strong-base-oxidized copper mesh | Deionized water | L = 45cm, D = 0.5cm | Fluororubber tube | Flexible cylindrical heat pipe | Fluororubber tube in the adiabatic section offered excellent flexibility and thermal stability |
| 6 | 2018 | Lee et al. | Copper mesh | Deionized water | L = 10.6cm, W = 3.6 | Flexible copper | Pure-metal-based sub-millimetre thick flexible flat heat pipe | Pure metal heat pipe with hydrophilic stent like mesh structure for improved performance |
| 7 | 2019 | Liu et al. | SS mesh covered with nanowires | Deionized water | L = 13cm, W = 6cm | Aluminum compound packing film | Flexible flat heat pipes | Polymeric heat pipe which can bend up to 1800 |
| 8 | 2020 | Huang et al. | Copper mesh | Deionized water | L = 22cm, D = 0.5cm | several stiﬀ copper tubes and a continuous ﬂexible polymer shell | Flexible heat pipe with multistage design | Multi stage heat pipe inspired from human spine structure |
| 9 | 2021 | Sugimoto et al. | Polytetrafluoroe-thylene mesh in evaporator | Ethanol | L = 8cm, W = 4.5cm | Polydimethylsiloxane | Flexible loop heat pipe | Highly flexible heat pipe for cooling wearable devices |
| 10 | 2022 | Alqahtani et al. |  | Ethanol and refrigerant FC-72 | L = 25cm, W = 10cm | Polypropylene | Flexible polypropylene pulsating heat pipe | Evaluated the bending performance of pulsating heat pipe |
| 11 | 2023 | Sun et al. | multilayer copper mesh and porous micro-pillar arrays | Deionized water | L = 11cm, W = 2.5cm | Aluminum plastic film | Ultrathin flexible heat pipes | Used a wick containing porous micro-pillar arrays, inspired from the design of Microsorum fortunei leaf |
| 12 | 2023 | Morgan et al. |  | Ethanol, water | D = 5cm (circular sheet) | Acrylic polymer | Flexible pulsating heat pipe | Studied the cooling performance of flexible pulsating heat pipe |

Flexible heat pipe as heat spreader

| Sl. No. | Year | Authors | Type of Wick | Working Fluid | Dimensions | Flexible material | Type of heat pipe | Features |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2017 | J Lim |  | HFE-7000 | L = 10.15cm, W = 6.5cm | Low-density polyethylene | Polymer-based flexible pulsating heat pipe | Flexible, lightweight and reliable heat spreader for foldable devices |
| 2 | 2018 | Lim et al. |  | HFE-7000 | L = 8.55cm, W = 5.34cm | Low-density polyethylene | Polymer-based flexible pulsating heat pipe | Flexible heat spreaders with superior thermal conductivity and extended durability |
| 3 | 2020 | Jung et al. |  | HFE-7000 | L = 8.55cm, W = 5.34cm | Polycarbonate sheet was used as a base material and flexible copper clad laminate as envelope | High-performance flexible pulsating heat pipe | A flexible, thin pulsating heat pipe possessing exceptional thermal efficiency and assured durability over extended periods. |

Flexible heat pipes for thermal management of batteries

| Sl. No. | Year | Authors | Type of Wick | Working Fluid | Dimensions | Flexible material | Type of heat pipe | Features |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2017 | Qu et al. | Grooves on evaporator and condenser | Deionized water | L = 107cm, D = 0.4cm | Fluororubber 246 | Hybrid flexible oscillating heat pipe | Demonstrates strong performance across various spatial configurations and bending conditions. |
| 2 | 2018 | Qu et al. | Grooves on evaporator and condenser | Deionized water | L = 107cm, D = 0.4cm | Fluororubber 246 | Hybrid flexible oscillating heat pipe | Studied the impact of the length of the adiabatic section on the overall performance. |
| 3 | 2023 | Clemens et al. |  | FC72, ethanol | L = 26cm, T = 0.15cm | Flat polypropylene sheets | Flexible pulsating heat pipe | Investigated the impact of bending and microgravity environments on performance. |

Flexible heat pipes for thermal management with multiple heating and cooling sources

| Sl. No. | Year | Authors | Type of Wick | Working Fluid | Dimensions | Flexible material | Type of heat pipe | Features |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2021 | Huang et al. | Copper mesh | Deionized water | Condenser=6.5cm, Evaporator =6.5cm, Adiabatic= 2.5cm | Polyurethane tubes | Y-shaped ﬂexible branch heat pipe | Flexible branch heat pipe may be ideal for heat dissipation applications with various or unfixed heating/cooling sources. |
| 2 | 2022 | Kang et al. | Wickless | Acetone | L = 36cm | Teflon tubes | Pulsating heat pipe | Soft heat pipes resembling squids, featuring multiple branches for heat transport. |

Flexible heat pipes for precise temperature control

| Sl. No. | Year | Authors | Type of Wick | Working Fluid | Dimensions | Flexible material | Type of heat pipe | Features |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2009 | Streckert et al. | Not disclosed | Ammonia | L = 135cm, D = 1cm | EDU Flex Hose | Flexible Variable Conductance Heat Pipe | An innovative heat pipe capable of managing high heat flux |
| 2 | 2009 | Leitch et al. |  | Ammonia | L = 135cm, D = 1cm | EDU Flex Hose | Graded-groove flexible Variable Conductance Heat Pipe | To meet stringent thermal requirements |

Flexible heat pipes as stick-on heat transfer device

| Sl. No. | Year | Authors | Type of Wick | Working Fluid | Dimensions | Flexible material | Type of heat pipe | Features |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2000 | Rosenfeld et al. | Copper felt pads | Not disclosed | T = 0.3cm | Polypropylene | Thin flexible flat heat pipe | Heat transfer surface conforms to the body being cooled |
| 2 | 2004 | Rosenfeld et al. | Copper felt pads | Not disclosed | T = 0.3cm | Polypropylene | Thin flexible flat heat pipe | Two metal foil layers were used to make leak proof |

Flexible heat pipes for cryogenic applications

| Sl. No. | Year | Authors | Type of Wick | Working Fluid | Dimensions | Flexible material | Type of heat pipe | Features |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1976 | Wright et al. | Composite SS Mesh | Methane & Oxygen | D = 0.95cm | Braided SS bellow | High performance flexible cryogenic heat pipes | Flexible heat transfer device designed for use in cryogenic environments |
| 2 | 2003 | Khrustalev et al. | SS mesh | Oxygen | Transport line length = 60cm | Long small diameter wickless transport lines | cryogenic loop heat pipe | Used as thermal connection between cryocooler and cooled components |

Miscellaneous applications of flexible heat pipes

| Sl. No. | Year | Authors | Type of Wick | Working Fluid | Dimensions | Flexible material | Type of heat pipe | Features |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1973 | Franklin et al. | Felt metal Wick | Methanol | L = 45.72cm, D = 1.27cm | Flexible bellow | flexible heat pipe employing external tube arteries in the adiabatic region | Removed the necessity for a constant wick presence throughout. |
| 2 | 1983 | Wolf | SS screen mesh | Ammonia | Not disclosed | SS bellow | Thermal switch employing flexible heat pipe | A thermal switch to control the flow of heat from heat source to heat sink |
| 3 | 2006 | Odhekar et al. | Sintered copper felt | Water | L = 27.62cm, D = 0.635cm | Flexible copper | Flexible cylindrical heat pipes | Shown that flexible heat pipes can use sintered copper felt wicks |
| 4 | 2011 | Odhekar et al. | Micro-fibrous sintered copper felt | Water | L = 27.62cm, D = 0.635cm | Flexible copper | Flexible cylindrical heat pipes | then compared the outcome to its copper equivalent benchmark |
| 5 | 2013 | Oshman et al. | sintered copper mesh | Water | L = 13cm, W = 7cm | Mylar | Flat, flexible, lightweight, polymer heat pipes | High-performance flat heat pipe with low thermal resistance |
| 6 | 2015 | Narayan et al. | Phosphor bronze screen mesh | Water | L = 27cm, D = 1cm | SS bellow | Flexible cylindrical heat pipe | Flexible heat pipe that absorbs vibration and mounting forces |
| 7 | 2017 | Jaipurkar et al. | Phosphor bronze screen mesh | Water | L = 27cm, D = 1cm | SS bellow | Flexible cylindrical heat pipe | Thermal and mechanical characterization of flexible heat pipe |
| 8 | 2021 | Udaybhanu et al. | sintered copper material | Water | L = 56cm | Not disclosed | Flexible heat cylindrical pipe | A numerical analysis to investigate the impact of different parameters on performance. |
| 9 | 2023 | Ordu et al. |  |  |  |  | Flexible Pulsating Heat Pipe | Proposed multi-criteria decision-making techniques for polymeric material selection |

[1]	Cuce E, Harjunowibowo D, Cuce PM. Renewable and sustainable energy saving strategies for greenhouse systems: A comprehensive review. Renew Sustain Energy Rev 2016;64:34–59. https://doi.org/10.1016/j.rser.2016.05.077.

[2]	Chan CW, Siqueiros E, Royapoor M, Roskilly AP. Heat utilisation technologies : A critical review of heat pipes. Renew Sustain Energy Rev 2015;50:615–27. https://doi.org/10.1016/j.rser.2015.05.028.

[3]	Faghri A. Heat pipe science and technology. First. United Kingdom: Taylor & Francis; 1995.

[4]	Chi S. Heat pipe theory and practice: a sourcebook 1976.

[5]	Gibbons MJ, Marengo M, Persoons T. A review of heat pipe technology for foldable electronic devices. Appl Therm Eng 2021;194:117087. https://doi.org/10.1016/j.applthermaleng.2021.117087.

[6]	Bliss FE, Jr. C. EG, Jr. SB. CONSTRUCTION AND TEST OF A FLEXIBLE HEAT PIPE. Sanders Assoc Inc, Nashua, NH (ASME HT/SpT-13 Sp Technol Heat Transf Conf 1970:1–7.

[7]	Odhekar D, Harris DK. Bendable Heat Pipes Using Sintered Metal Felt Wicks. Front Heat Pipes 2011;2. https://doi.org/10.5098/fhp.v2.2.3002.

[8]	SAASKI E, WRIGHT J. A flexible cryogenic heat pipe. 10th Thermophys. Conf., Reston, Virigina: American Institute of Aeronautics and Astronautics; 1975. https://doi.org/10.2514/6.1975-658.

[9]	WRIGHT J, BRENNAN P, MCCREIGHT C. Development and test of two flexible cryogenic heat pipes. 11th Thermophys. Conf., Reston, Virigina: American Institute of Aeronautics and Astronautics; 1976. https://doi.org/10.2514/6.1976-478.

[10]	Shaubach RM, Gernert NJ. High Performance Flexible Heat Pipes. AIAA Pap 1985:0–6. https://doi.org/10.2514/6.1985-1085.

[11]	Odhekar DD, Harris DK. Experimental investigation of bendable heat pipes using sintered copper felt wick. Thermomechanical Phenom Electron Syst -Proceedings Intersoc Conf 2006;2006:570–7. https://doi.org/10.1109/ITHERM.2006.1645396.

[12]	Glass DE, Stevens JC, Raman V V. Flexible heat pipes for a lightweight spacecraft radiator. J Spacecr Rockets 1999;36:711–8. https://doi.org/10.2514/2.3484.

[13]	Liu C, Li Q, Fan D. Fabrication and performance evaluation of flexible flat heat pipes for the thermal control of deployable structure. Int J Heat Mass Transf 2019;144:118661. https://doi.org/10.1016/j.ijheatmasstransfer.2019.118661.

[14]	Rosenfeld JH, Gernert NJ, Sarraf DB, Wollen PJ, Surina FC, Fale JE. Improved Thin, Flexible Heat Pipes. NASA Tech Briefs, Febr 2004 2004.

[15]	Oshman C, Shi B, Li C, Yang R, Lee YC, Peterson GP, et al. The development of polymer-based flat heat pipes. J Microelectromechanical Syst 2011;20:410–7. https://doi.org/10.1109/JMEMS.2011.2107885.

[16]	Wu G-W, Chen S-L, Shih W-P. Lamination and Characterization of a Polyethylene-Terephthalate Flexible Micro Heat Pipe. Front Heat Pipes 2012;3:1–5. https://doi.org/10.5098/fhp.v3.2.3003.

[17]	Hsieh SS, Yang YR. Design, fabrication and performance tests for a polymer-based flexible flat heat pipe. Energy Convers Manag 2013;70:10–9. https://doi.org/10.1016/j.enconman.2013.02.011.

[18]	Yang C, Song C, Shang W, Tao P, Deng T. Flexible heat pipes with integrated bioinspired design. Prog Nat Sci Mater Int 2015;25:51–7. https://doi.org/10.1016/j.pnsc.2015.01.011.

[19]	Yang C, Chang C, Song C, Shang W, Wu J, Tao P, et al. Fabrication and performance evaluation of flexible heat pipes for potential thermal control of foldable electronics. Appl Therm Eng 2016;95:445–53. https://doi.org/10.1016/j.applthermaleng.2015.11.078.

[20]	Lim J, Kim SJ. Fabrication and experimental evaluation of a polymer-based flexible pulsating heat pipe. Energy Convers Manag 2018;156:358–64. https://doi.org/10.1016/j.enconman.2017.11.022.

[21]	Miyazaki Y. Cooling of Notebook PCs by Flexible Oscillating Heat Pipes. Proc. IPACK2005,  ASME InterPACK ’05, ASMEDC; 2005, p. 65–9. https://doi.org/10.1115/IPACK2005-73055.

[22]	Shiraishi M, Mochizuki M, Sugihara S, Yamagishi Y, Watanabe F. Snow removal and de-icing using long flexible heat pipes. Ann Glaciol 1993;18:328–9. https://doi.org/10.3189/S0260305500011812.

[23]	Wang X, Fan H, Zhu Y, Zhu M. Heat Transfer Simulation and Analysis of Ice and Snow Melting System Using Geothermy by Super-long Flexible Heat Pipes. Energy Procedia 2017;105:4724–30. https://doi.org/10.1016/j.egypro.2017.03.1026.

[24]	Wang X, Zhu Y, Zhu M, Zhu Y, Fan H, Wang Y. Thermal analysis and optimization of an ice and snow melting system using geothermy by super-long flexible heat pipes. Appl Therm Eng 2017;112:1353–63. https://doi.org/10.1016/j.applthermaleng.2016.11.007.

[25]	Wang X, Wang Y, Wang Z, Liu Y, Zhu Y, Chen H. Simulation-based analysis of a ground source heat pump system using super-long flexible heat pipes coupled borehole heat exchanger during heating season. Energy Convers Manag 2018;164:132–43. https://doi.org/10.1016/j.enconman.2018.03.001.

[26]	Liu H, Wang X, Zhu Y. Experimental investigation on temperature response and heat transfer characteristics of super-long flexible heat pipe for geothermal utilization. Int Conf Appl Energy 2020:1–5.

[27]	Liu H, Wang X, Wang K, Zhu Y, Zhu Y. Numerical analysis of ground temperature response characteristics of a space-heating ground source heat pump system by utilizing super-long flexible heat pipes for heat extraction. Energy Build 2021;244:110991. https://doi.org/10.1016/j.enbuild.2021.110991.

[28]	Liu H, Wang X, Gan W, Gao H, Yao H, Wang Y, et al. Experimental investigation on extraction of shallow geothermal energy by flexible two-phase closed thermosyphons: Effect of geometric parameters. Energy Build 2023;298:113515. https://doi.org/10.1016/j.enbuild.2023.113515.

[29]	Meseguer J, Pérez-Grande I, Sanz-Andrés A. Spacecraft thermal control. Elsevier; 2012. https://doi.org/10.1533/9780857096081.

[30]	Sunil Kumar S, Nayak V, Venkateshan SP. Optimum finned space radiators. Int J Heat Fluid Flow 1993;14:191–200. https://doi.org/10.1016/0142-727X(93)90028-L.

[31]	Edelstein F. Deployable Heat Pipe Radiator 1975.

[32]	Merrigan M, Keddy E, Sena J, Elder M. Heat pipe technology development for high temperature space radiator applications 1984.

[33]	Hwangbo H, Joost TE. A flexible variable conductance heat pipe design for temperature control of spacecraft equipment. AIAA 23rd Thermophys Plasmadynamics Lasers Conf 1988 1998. https://doi.org/10.2514/6.1988-2680.

[34]	Trujillo V, Keddy E, Merrigan M. High-temperature, deployable, membrane heat-pipe radiator element; demonstration and status 1989.

[35]	Fale JE, Gernert NJ, Cornwell JD. Flexible heat pipe radiator. SAE Tech Pap 1998. https://doi.org/10.4271/981682.

[36]	Krotiuk WJ, Crowley CJ, Rozzi JC. Propylene loop heat pipe with a lightweight, flexible, deployable radiator. AIP Conf. Proc., AIP; 2002, p. 37–48. https://doi.org/10.1063/1.1449706.

[37]	Abe Y, Tanaka K, Nakagawa M, Di Francescantonio N, Savino R, Iwasaki A. Flexible wickless heat pipes radiator with self-rewetting fluids. Collect Tech Pap - 9th AIAA/ASME Jt Thermophys Heat Transf Conf Proc 2006;1:666–75. https://doi.org/10.2514/6.2006-3105.

[38]	Tanaka K, Abe Y, Nakagawa M, Piccolo C, Savino R. Low-gravity experiments of lightweight flexible heat pipe panels with self-rewetting fluids. Ann N Y Acad Sci 2009;1161:554–61. https://doi.org/10.1111/j.1749-6632.2008.04333.x.

[39]	Ambrose J. Flex Heat Pipe East-West Deployable Radiator. Microgravity Sci Technol 2019:311–6. https://doi.org/10.1007/s12217-019-9693-2.

[40]	Bernardin JD. The Performance of Methanol and Water Heat Pipes for Electronics Cooling Applications in Spacecraft Instrumentation. Heat Transf. Vol. 4, ASMEDC; 2005, p. 669–79. https://doi.org/10.1115/HT2005-72057.

[41]	Thienel L, Lewis MR, Brennan PJ, Buchko M, Glaister D, Stoyanof M. Design and performance of the cryogenic flexible diode heat pipe (CRYOFD) flight experiment. SAE Tech Pap 1998. https://doi.org/10.4271/981583.

[42]	Gernert NJ. Flexible heat pipe cold plate. Final Rep Nav Air Dev Center, Warm PA 1989;89067–60.

[43]	Gernert N, Sarraf D, Steinberg M. Flexible heat pipe cold plates for aircraft thermal control. SAE Tech Pap 1991. https://doi.org/10.4271/912105.

[44]	Thomas SK, Yerkes KL. Quasi-Steady-State Performance of a Heat Pipe Subjected to Transient Acceleration Loadings. J Thermophys Heat Transf 1997;11:306–9. https://doi.org/10.2514/2.6239.

[45]	Hoang TT, O’Connell TA, Khrustalev DK. Development of a flexible advanced loop heat pipe for across-gimball cryocooling. Cryog Opt Syst Instruments X 2003;5172:68. https://doi.org/10.1117/12.501493.

[46]	Hoang TT, O’Connell TA. Performance demonstration of flexible advanced loop heat pipe for across-gimbal cryocooling. Collect Tech Pap - 3rd Int Energy Convers Eng Conf 2005;2:796–806. https://doi.org/10.2514/6.2005-5590.

[47]	Thayer J, Hall G. Flexible air cooled loop heat pipe for high density avionics cooling. Collect Tech Pap - 9th AIAA/ASME Jt Thermophys Heat Transf Conf Proc 2006;3:1493–504. https://doi.org/10.2514/6.2006-3413.

[48]	Der O, Marengo M, Bertola V. A low cost, flexible pulsating heat pipe technology. Proc Therm Fluids Eng Summer Conf 2018;2018-March:321–7. https://doi.org/10.1615/TFEC2018.ces.021455.

[49]	Rathod J, Conference VL-II, 2019  undefined. Design, Development and Testing of SS/Acetone Screen Mesh Flexible Heat Pipe. JournalsLibraryTorontomuCaJ Rathod, V LakheraICTEA Int Conf Therm Eng 2019•journalsLibraryTorontomuCa n.d.

[50]	Clemens F, Georgoulas A, … NM-17th I, 2023  undefined. Thermal characterisation of a flexible pulsating heat pipe in different gravity conditions. Res Clemens, A Georg N Miche, M Marengo, M Bernagozzi, G Sercey, F Bozzoli17th Int Heat Transf Conf 2023•researchBrightonAcUk n.d.

[51]	Nellis GF. Thermal Resistance Measurements for Flexible Straps at Cryogenic Temperatures. AIP Conf. Proc., AIP; 2004, p. 651–8. https://doi.org/10.1063/1.1774739.

[52]	7.0 Thermal Control - NASA n.d. https://www.nasa.gov/smallsat-institute/sst-soa/thermal-control/#7.2.3 (accessed October 31, 2023).

[53]	Ralphs M, Sinfield M, Felt M. High Performance Thermal Straps for a Full Range of Application Temperatures. Cryocoolers 2021;21:1–10.

[54]	Wilson MR, Hengeveld DW, Taft BS. Advanced Manufacturing of Flexible Oscillating Heat Pipes for Next-Generation Thermal Straps. 47th Int Conf Environ Syst  2017.

[55]	Iwata N, Miyazaki Y, Yasuda S, Ogawa H. Thermal performance and flexibility evaluation of metallic micro oscillating heat pipe for thermal strap. Appl Therm Eng 2021;197:117342. https://doi.org/10.1016/j.applthermaleng.2021.117342.

[56]	Nagano H, Mizutani T, Kajiayama S, Akizuki Y, Machida Y, Directorate D. Testing and Evaluation of Ultra-Thin Loop Heat Pipe as Lightweight Flexible Thermal Strap for Spacecraft 2022:1–7.

[57]	Gabsi I, Maalej S, Zaghdoudi MC. Thermal performance modeling of loop heat pipes with flat evaporator for electronics cooling. Microelectron Reliab 2018;84:37–47. https://doi.org/10.1016/j.microrel.2018.02.023.

[58]	Chen X, Ye H, Fan X, Ren T, Zhang G. A review of small heat pipes for electronics. Appl Therm Eng 2016;96:1–17. https://doi.org/10.1016/j.applthermaleng.2015.11.048.

[59]	Zohuri B. Heat pipe design and technology: Modern applications for practical thermal management, second edition. Heat Pipe Des Technol Mod Appl Pract Therm Manag Second Ed 2016:1–513. https://doi.org/10.1007/978-3-319-29841-2/COVER.

[60]	Kobayashi T, Ogushi T, Haga S, Ozaki E, Fujii M. Heat transfer performance of a flexible looped heat pipe using R134a as a working fluid: Proposal for a method to predict the maximum heat transfer rate of FLHP. Heat Transf - Asian Res 2003;32:306–18. https://doi.org/10.1002/htj.10093.

[61]	Vasiliev L, Grakovich L, Rabetsky M, Vasiliev L. INNOVATIVE HEAT PIPES AND NANOTECHNOLOGIES. Proc CONV-14 Int Symp Convect Heat Mass Transf 2014:953–63. https://doi.org/10.1615/ICHMT.2014.INTSYMPCONVHEATMASSTRANSF.710.

[62]	Shi B, Wang YB, Shan YJ. An Experimental Investigation of Thermal Performance of a Polymer-Based Flat Heat Pipe. Heat Transf - Asian Res 2016;45:746–57. https://doi.org/10.1002/htj.21187.

[63]	Dai X, Tang Y, Liu T, Wang S. Experimental investigation on the thermal characteristics of ultra-thin flattened heat pipes with bending angles. Appl Therm Eng 2020;172:115150. https://doi.org/10.1016/j.applthermaleng.2020.115150.

[64]	Udaybhanu G, A R, Chakraborty S. Computational investigation of the thermohydraulic performance of a flexible heat pipe for various bending angle, filling ratios and materials. Proceeding Proc. 26thNational 4th Int. ISHMT-ASTFE Heat Mass Transf. Conf. December 17-20, 2021, IIT Madras, Chennai-600036, Tamil Nadu, India, Connecticut: Begellhouse; 2021, p. 2507–13. https://doi.org/10.1615/IHMTC-2021.3790.

[65]	Yang X, Liu T, Yan W, Wang S. Design, fabrication and heat transfer performance study of a novel flexible flat heat pipe. Int Commun Heat Mass Transf 2023;142:106673. https://doi.org/10.1016/j.icheatmasstransfer.2023.106673.

[66]	Han Z, Chang C. Fabrication and Thermal Performance of a Polymer-Based Flexible Oscillating Heat Pipe via 3D Printing Technology. Polymers (Basel) 2023;15. https://doi.org/10.3390/polym15020414.

[67]	Gibbons MJ, Marengo M, Persoons T. A review of heat pipe technology for foldable electronic devices. Appl Therm Eng 2021;194:117087. https://doi.org/10.1016/j.applthermaleng.2021.117087.

[68]	Tang H, Tang Y, Wan Z, Li J, Yuan W, Lu L, et al. Review of applications and developments of ultra-thin micro heat pipes for electronic cooling. Appl Energy 2018;223:383–400. https://doi.org/10.1016/j.apenergy.2018.04.072.

[69]	Lee D, Byon C. Fabrication and characterization of pure-metal-based submillimeter-thick flexible flat heat pipe with innovative wick structures. Int J Heat Mass Transf 2018;122:306–14. https://doi.org/10.1016/j.ijheatmasstransfer.2018.01.135.

[70]	Huang J, Zhou W, Xiang J, Liu C, Gao Y, Li S, et al. Development of novel flexible heat pipe with multistage design inspired by structure of human spine. Appl Therm Eng 2020;175:115392. https://doi.org/10.1016/j.applthermaleng.2020.115392.

[71]	Ju YS. Thermal management and control of wearable devices. IScience 2022;25:104587. https://doi.org/10.1016/j.isci.2022.104587.

[72]	Sugimoto K, Alasli A, Ueno A, Nagano H. Design and Fabrication of Flexible Two-Phase Heat Transport Device for Wearable Interfaces. Proc IEEE Int Conf Micro Electro Mech Syst 2021;2021-January:95–8. https://doi.org/10.1109/MEMS51782.2021.9375378.

[73]	Alqahtani AA, Edwardson S, Marengo M, Bertola V. Performance of flat-plate, flexible polymeric pulsating heat pipes at different bending angles. Appl Therm Eng 2022;216:118948. https://doi.org/10.1016/j.applthermaleng.2022.118948.

[74]	Sun Y, Liang F, Tang H, Tang Y, Zhang S, Chua KJ. Ultrathin flexible heat pipes with Microsorum fortunei structural-like wick for cooling flexible electronic devices. Int J Heat Mass Transf 2023;202:123743. https://doi.org/10.1016/j.ijheatmasstransfer.2022.123743.

[75]	Morgan N, Hundley D, Baron M, Pichardo M, Putnam SA. Fabrication and Testing of Flexible Pulsating Heat Pipes. Intersoc Conf Therm Thermomechanical Phenom Electron Syst ITHERM 2023;2023-May:1–7. https://doi.org/10.1109/ITherm55368.2023.10177672.

[76]	Wang M, Cui W, Hou Y. Thermal spreading resistance of grooved vapor chamber heat spreader. Appl Therm Eng 2019;153:361–8. https://doi.org/10.1016/j.applthermaleng.2019.03.025.

[77]	Lim J. Experimental study on the fabrication and thermal performance evaluation of the polymer-based flexible pulsating heat pipe. Korea Advanced Institute of Science and Technology, 2017.

[78]	Jung C, Lim J, Kim SJ. Fabrication and evaluation of a high-performance flexible pulsating heat pipe hermetically sealed with metal. Int J Heat Mass Transf 2020;149. https://doi.org/10.1016/j.ijheatmasstransfer.2019.119180.

[79]	Smith J, Singh R, Hinterberger M, Mochizuki M. Battery thermal management system for electric vehicle using heat pipes. Int J Therm Sci 2018;134:517–29. https://doi.org/10.1016/j.ijthermalsci.2018.08.022.

[80]	Abdelkareem MA, Maghrabie HM, Abo-Khalil AG, Adhari OHK, Sayed ET, Radwan A, et al. Thermal management systems based on heat pipes for batteries in EVs/HEVs. vol. 51. Elsevier Ltd; 2022. https://doi.org/10.1016/j.est.2022.104384.

[81]	Bernagozzi M, Georgoulas A, Miché N, Marengo M. Heat pipes in battery thermal management systems for electric vehicles: A critical review. Appl Therm Eng 2023;219. https://doi.org/10.1016/j.applthermaleng.2022.119495.

[82]	Qu J, Li X, Cui Y, Wang Q. Design and experimental study on a hybrid flexible oscillating heat pipe. Int J Heat Mass Transf 2017;107:640–5. https://doi.org/10.1016/j.ijheatmasstransfer.2016.11.076.

[83]	Qu J, Wang C, Li X, Wang H. Heat transfer performance of flexible oscillating heat pipes for electric/hybrid-electric vehicle battery thermal management. Appl Therm Eng 2018;135:1–9. https://doi.org/10.1016/j.applthermaleng.2018.02.045.

[84]	Tang H, Tang Y, Li J, Sun Y, Liang G, Peng R. Experimental investigation of the thermal performance of heat pipe with multi-heat source and double-end cooling. Appl Therm Eng 2018;131:159–66. https://doi.org/10.1016/j.applthermaleng.2017.12.006.

[85]	Huang J, Xiang J, Chu X, Sun W, Liu R, Ling W, et al. Thermal performance of flexible branch heat pipe. Appl Therm Eng 2021;186:116532. https://doi.org/10.1016/j.applthermaleng.2020.116532.

[86]	Kang Z, Jiang S, Hong Y, Fan J. Squid‐like soft heat pipe for multiple heat transport. Droplet 2022;1:182–91. https://doi.org/10.1002/dro2.25.

[87]	Cleary M, North MT, Van Lieshout M, Brooks DA, Grimes R, Hodes M. Reduced power precision temperature control using variable conductance heat pipes. IEEE Trans Components, Packag Manuf Technol 2013;3:2048–58. https://doi.org/10.1109/TCPMT.2013.2270286.

[88]	Streckert L, Ambrose J. Steady state performance results for a flexible high-performance variable conductance heat pipe. SAE Tech Pap 2009;4970. https://doi.org/10.4271/2009-01-2502.

[89]	Leitch R, Ambrose J. Transient response of a flexible high-performance variable conductance heat pipe. SAE Tech Pap 2009;4970. https://doi.org/10.4271/2009-01-2500.

[90]	Rosenfeld JH, Gernert NJ, Sarraf DB, Wollen P, Surina F, Fale J. FLEXIBLE HEAT PIPE. US 6,446,706 B1, 2002.

[91]	Jouhara H, Chauhan A, Guichet V, Delpech B, Abdelkareem MA, Olabi AG, et al. Low-temperature heat transfer mediums for cryogenic applications. J Taiwan Inst Chem Eng 2023;148:104709. https://doi.org/10.1016/j.jtice.2023.104709.

[92]	Dunn P, Reay DA. Heat pipes. Third edition. 1982.

[93]	Zohuri B. Functionality, Advancements and Industrial Applications of Heat Pipes. Elsevier; 2020. https://doi.org/10.1016/C2019-0-01065-5.

[94]	Khrustalev D. Cryogenic Loop Heat Pipes as Flexible Thermal Links for Cryocoolers 2003:709–16.

[95]	Franklin JL, Pogson JT. External tube artery flexible heat pipe. US3913665A, 1973.

[96]	Wolf D. Heat pipe thermal switch. 4,402,358, 1983.

[97]	Oshman C, Li Q, Liew L-A, Yang R, Bright VM, Lee YC. Flat flexible polymer heat pipes. J Micromechanics Microengineering 2013;23:015001. https://doi.org/10.1088/0960-1317/23/1/015001.

[98]	Narayan A, Jaipurkar T, Khandekar S, Paralikar S. Development and Testing of Flexible Heat Pipes. Proc 23rd Natl Heat Mass Transf Conf 1st Int ISHMT-ASTFE Heat Mass Transf Conf 2015:1–6.

[99]	Jaipurkar T, Kant P, Khandekar S, Bhattacharya B, Paralikar S. Thermo-mechanical design and characterization of flexible heat pipes. Appl Therm Eng 2017;126:1199–208. https://doi.org/10.1016/j.applthermaleng.2017.01.036.

[100]	Udaybhanu G, Rajalingam A, Chakraborty S. Thermo-hydraulic Investigation of the flexible heat pipe for various bending angle , filling ratios and materials. Proc 26th Natl 4th Int ISHMT-ASTFE Heat Mass Transf Conf 2021:1–6.

[101]	Ordu M, Der O. Polymeric Materials Selection for Flexible Pulsating Heat Pipe Manufacturing Using a Comparative Hybrid MCDM Approach. Polymers (Basel) 2023;15:1–20. https://doi.org/10.3390/polym15132933.

[102]	Sudhan ALS, Solomon AB, Sunder S. Heat transport limitations and performance enhancement of anodized grooved heat pipes charged with ammonia under gravity and anti-gravity condition. Appl Therm Eng 2022;200:117633. https://doi.org/10.1016/j.applthermaleng.2021.117633.

[103]	Chen Z, Li Y, Zhou W, Deng L, Yan Y. Design, fabrication and thermal performance of a novel ultra-thin vapour chamber for cooling electronic devices. Energy Convers Manag 2019;187:221–31. https://doi.org/10.1016/J.ENCONMAN.2019.03.038.

[104]	Jyothi Sankar PR, Venkatachalapathy S, Santhosh Kumar MC. Effect of hydrophilic coating on mesh wicks used in heat pipes. Https://DoiOrg/101080/0267084420191695370 2020;36:680–6. https://doi.org/10.1080/02670844.2019.1695370.

[105]	Sandeep SS, Prakash SB. Design of heat pipe based on capillary and entrainment limitations, AIP Conference Proceedings 2200, 020069 (2019); 2019, p. 020069. https://doi.org/10.1063/1.5141239.

[106]	Kempers R, Ewing D, Ching CY. Effect of number of mesh layers and fluid loading on the performance of screen mesh wicked heat pipes. Appl Therm Eng 2006;26:589–95. https://doi.org/10.1016/J.APPLTHERMALENG.2005.07.004.

[107]	Ranjan R, Patel A, Garimella S V., Murthy JY. Wicking and thermal characteristics of micropillared structures for use in passive heat spreaders. Int J Heat Mass Transf 2012;55:586–96. https://doi.org/10.1016/J.IJHEATMASSTRANSFER.2011.10.053.

[108]	Adera S, Antao D, Raj R, Wang EN. Design of Micropillar Wicks for Thin-Film Evaporation 2016.

[109]	Septiadi WN, Iswari GA, Sudarsana PB, Putra GJP, Febraldo D, Putra N, et al. Effect of Al2O3 and TiO2 nano-coated wick on the thermal performance of heat pipe. J Therm Anal Calorim 2022;147:6193–205. https://doi.org/10.1007/S10973-021-11034-4/METRICS.

[110]	Huang X, Franchi G. Design and fabrication of hybrid bi-modal wick structure for heat pipe application. J Porous Mater 2008;15:635–42. https://doi.org/10.1007/S10934-007-9143-1/METRICS.

[111]	Dai X, Tran L, Yang F, Shi B, Yang R, Lee YC, et al. Characterization of Hybrid-Wicked Copper Heat Pipe 2011. https://doi.org/10.1115/AJTEC2011-44088.

[112]	Famouri M, Carbajal G, Li C. Transient analysis of heat transfer and fluid flow in a polymer-based Micro Flat Heat Pipe with hybrid wicks. Int J Heat Mass Transf 2014;70:545–55. https://doi.org/10.1016/J.IJHEATMASSTRANSFER.2013.11.019.

[113]	Narendra Babu N, Kamath H. Materials used in Heat Pipe. Mater Today Proc 2015;2:1469–78. https://doi.org/10.1016/J.MATPR.2015.07.072.

[114]	Dominguez Espinosa FA, Peters TB, Brisson JG. Effect of fabrication parameters on the thermophysical properties of sintered wicks for heat pipe applications. Int J Heat Mass Transf 2012;55:7471–86. https://doi.org/10.1016/J.IJHEATMASSTRANSFER.2012.07.037.

[115]	Han C, He L, Tian Z, Xu B, Chen Z. Study of heat dissipation characteristics of loop heat pipe with heat sink of composite material. Appl Therm Eng 2022;200:117572. https://doi.org/10.1016/J.APPLTHERMALENG.2021.117572.

[116]	Franchi G, Huang X. Development of Composite Wicks for Heat Pipe Performance Enhancement. Heat Transf Eng 2008;29:873–84. https://doi.org/10.1080/01457630802125740.