I. INTRODUCTION

The importance of power factor correction (PFC) in any electrical system has evolved because of its meaningful effects on energy efficiency and power correction. The increase of inductive loads across the reactive power network has had effects on energy voltage drop and system performance. Thus, the energy losses and voltage stability in distribution networks make the lower-range PFC networks compliment energy efficiency. The PFC networks have used microcontrollers to make systems more flexible. Real-time monitoring systems designed to shift between capacitor banks dynamically balance power factor correction and PFC networks based on the load. Embedded systems provide these networks with the flexibly designed command and control system to PFC networks to combat the change in electrical demand. Studies have shown the coupling of PFC with other network systems, especially renewable networks like photovoltaic (PV) systems, greatly enhances the network reliability, power factor and overall PFC energy efficiency.

This work presents a Real-Time Power Factor

Correction Controller employing an Arduino-based system

that continuously observes the power factor and adaptive

controls a capacitor bank. To achieve optimum reactive

power compensation, the system employs an automatic relay

switching control in combination with current and potential

transformers for real-time measurement. The objective is to

maintain the power factor close to unity to enhance energy

efficiency, reduce low power factor fines, and ensure stable

voltage regulation in power grids. The objective of the

proposed system is to provide a scalable and affordable realtime

solution to power factor compensation. This approach

minimizes the losses of energy, enhances flexibility in

response to changes in load, and offers an implementable

solution suitable for industrial and domestic use with the help

of an Arduino-based embedded platform.

The project's objective is to create real-time power

factor correction controller using Arduino. The following

objectives for our project are listed below:

✓ Develop a real-time monitoring system using

Arduino to continuously assess the power factor of

inductive loads through voltage and current

measurements.

✓ Design an automatic control mechanism that

dynamically adjusts the capacitive bank to optimize

the power factor based on the measured data.

✓ Utilize current and potential transformers for

accurate measurement of electrical parameters,

ensuring precise monitoring and control.

✓ Analyze the effectiveness of the implemented power

factor correction strategy in enhancing energy

efficiency and reducing reactive power losses.

✓ Provide a scalable and practical solution suitable for both

industrial and residential electrical systems, contributing

to improved voltage regulation and overall power quality.

II LITERATURE SURVEY

Power factor correction (PFC) is a critical aspect of

modern electrical engineering, as it plays a significant role in

enhancing energy efficiency and maintaining the quality of

power supplied to various systems. Bisanovic et al. (2018)

presented a method for reactive power control of capacitor

banks in distribution and industrial networks. Their

approach highlights the importance of managing reactive

power to optimize voltage levels and improve system

performance. By focusing on capacitor banks, the study

underscores the potential for automatic adjustments in

power factor to alleviate issues arising from reactive power

demand. In recent developments, Haneef et al. (2024)

introduced a stochastic optimal placement method for

photovoltaic (PV) systems, emphasizing command-based

power control for voltage and PFC. Their work explores the

integration of renewable energy sources, which is

increasingly relevant in contemporary power systems. This

study demonstrates the effectiveness of optimization

algorithms in achieving a desirable power factor, especially in

hybrid systems that combine traditional and renewable

energy sources. The application of PFC in specialized

circuits has also been examined by Fusic et al. (2024), who

focused on efficient power factor correction in single-phase

AC–DC boost converters. Their findings indicate that

enhancing PFC in electric bicycle battery charging systems

can lead to improved energy utilization, highlighting the

importance of addressing PFC at the component level within

electronic devices. Ali (2013) contributed to the

understanding of microcontroller-based PFC systems by

designing and implementing a solution that utilizes

capacitor banks alongside load monitoring techniques. This

study exemplifies the practical applications of

microcontrollers, such as Arduino, in automating power

factor adjustments based on real-time data, thereby reducing

energy losses and enhancing operational efficiency. Further

advancements in PFC systems can be seen in the work of

Díaz et al. (2023), who developed a smart energy meter

capable of synchronous monitoring within distributed

systems. Their research indicates that real-time data

acquisition and analysis can facilitate effective power factor

correction strategies, offering insights into user behavior and

energy consumption patterns. Cosic and Sadarangani (2016)

explored the dynamic performance of induction machines

with a novel converter-fed rotor concept, contributing to the

understanding of how PFC techniques can be integrated into

motor control applications. Their findings suggest that

innovative converter designs can significantly enhance

power factor performance, which is crucial for various

industrial applications. Kushwaha and Singh (2019) focused

on power factor improvement in modified bridgeless

Landsman converters used for electric vehicle (EV) battery

charging. Their research demonstrates that PFC methods can

be effectively tailored to specific applications, thereby

optimizing energy efficiency in the rapidly growing EV

market.Lo et al. (2008) examined grid-connected

photovoltaic systems with PFC capabilities, highlighting the

synergy between renewable energy integration and power

factor improvement. This study emphasizes the role of PFC

in facilitating the adoption of sustainable energy solutions.

Zandabad and Fazel (2024) introduced a single-stage variable

output AC-DC converter that employs PFC and softswitching

techniques for renewable energy integration. Their

innovative approach reflects the ongoing efforts to enhance

energy conversion systems while maintaining a high power

factor. Finally, Bhati and Kalla (2019) investigated a power

factor corrected Y-cell modified boost converter for battery

charging applications in electric vehicles. Their research

contributes to the growing body of knowledge on optimizing

PFC in power electronics, showcasing practical

implementations for enhancing battery charging efficiency.

This literature survey highlights the diverse approaches and

technologies employed in power factor correction,

underscoring the significance of real-time monitoring and

control systems. The proposed Real-Time Power Factor

Correction Controller using Arduino aims to build on these

findings, providing an innovative solution for optimizing

power factor in response to dynamic load conditions while

contributing to the ongoing advancements in electrical

engineering

III EXISTING METHOD

One commonly used approach in power factor

correction systems is the Automatic Switched Capacitor

Bank. This method involves the use of a capacitor bank

divided into multiple stages.

In power electronics, Fusic et al. (2024) investigated

PFC in single-phase AC–DC boost converters for use in

electric bicycle charging systems, demonstrating how

optimizations at the component level enhance system

efficiency. Ali (2013) developed a microcontroller-based

PFC system with capacitor banks, demonstrating how realtime

load monitoring can be employed to minimize energy

losses and maximize system performance.

For distributed power systems, Díaz et al. (2023)

created a smart energy meter for synchronous monitoring

with real-time data acquisition for optimal PFC techniques.

Cosic and Sadarangani (2016) investigated the dynamic

performance of induction machines.

In addition, Kushwaha and Singh (2019) researched power

factor improvement in modified bridgeless Landsman

converters for electric vehicle (EV) charging, proving the

significance of application-specific PFC techniques. Lo et al.

(2008) explored the interfacing of grid-connected PV systems

with PFC, further supporting the significance of sustainable

energy solutions for power factor performance improvement.

Zandabad and Fazel (2024) presented a single-stage AC-DC

converter utilizing PFC and soft-switching, proving innovative

methods for integrating renewable energy.

IV. PROPOSED SYSTEM

The system is designed for real-time power factor

correction using a combination of transformers,

microcontroller, relays, and capacitors. It begins with a

single-phase supply connected to both a potential transformer

and a current transformer.

The potential transformer steps down the voltage to

a level suitable for measurement, while the current

transformer measures the load current. Both transformers

provide inputs to the microcontroller, which is programmed

to calculate the power factor by analyzing the phase

difference between the voltage and current signals.

The Fig [1] shown the Block diagram of Real-time power

factor correction controller using Arduino

Based on the power factor readings, the microcontroller

activates a relay connected to a capacitor bank. The relay

allows selective engagement or disengagement of capacitors

in the bank to adjust the reactive power, thus improving the

overall power factor. When the power factor falls below a

specified threshold, the microcontroller switches in

additional capacitors to compensate for the reactive load. An

LCD display is included in the system, which provides a realtime

view of the power factor, voltage, and current

measurements for monitoring purposes.

This configuration helps maintain a power factor close to

unity by dynamically adjusting capacitance based on the load,

ensuring efficient power usage and minimizing energy losses

in inductive loads.

Fig. [2], shows the circuit diagram of Real-time power factor

correction controller using Arduino and The Real-Time

Power Factor Correction Controller system operates by

utilizing a potential transformer (PT) and a current

transformer (CT) to monitor essential electrical parameters.

The PT steps down the high AC voltage to a safer level

suitable for processing, while the CT measures the current

flowing through the load, enabling the system to dynamically

respond to changing load conditions. The o u t p u t from the

PT passes through a bridge rectifier, whichconverts AC to pulsating DC. A filter circuit smooths this DC

signal, and a voltage regulator then provides a stable 5V

supply to the Arduino Uno. The Arduino continuously

monitors the voltage and current levels, measuring the phase

difference between the two to calculate the power factor. This

phase difference measurement allows for precise calculation

of the power factor, providing real-time insight into the

system's performance. If the power factor deviates from the

desired range, the Arduino activates relays to switch a

capacitor bank in or out of the circuit as needed. This

adjustment compensates for reactive power, bringing the

power factor closer to unity, which helps reduce energy

VI RESULTS AND DISCUSSION

The Real-Time Power Factor Correction (APFC) system was

simulated with Proteus 8.13 to confirm its capability in

dynamically improving the power factor. The simulation was

done using an AC power supply, current and potential

transformers, Arduino-based controller, capacitor bank, and

switching relays. In Fig 3 Shown the initial power factor of 0.6.

readings easily. Additionally, the system proved to be costeffective,

with reductions in reactive power penalties and

energy costs leading to a favorable return on investment. The

automated nature of the APFC minimized the need for

manual intervention, further lowering operational costs.

Capacitive bank ON.

In Fig 4 show the effectively the power factor

improved to 0.95–0.978 by automatically turning on

capacitors of 0.44 kVAR as required.These results highlight

the critical importance of power factor improvement in

industrial and household applications, particularly given the

rise in inductive loads due to industrialization. Overall, the

findings indicate that microcontroller-based embedded

technology can significantly enhance power management

systems, making this APFC device a valuable solution for

improving energy efficiency across various sectors The Fig.4

Represents the Power Factor increased by using the

capacitive bank.

VII CONCLUSION

Despite the advantages of the proposed system,

there are certain practical limitations. The efficiency of

corrections can be affected by minor inaccuracies injected

into power factor computations by the accuracy of current and

potential transformers. While the system is well-suited to

small and medium applications, complex hardware

adjustments may be required for use in high-power industrial

systems. To improve power distribution and enable

automated energy management, future research can explore

how the system can be interfaced with smart grids. To

enhance sustainability and energy efficiency, hybrid energy

systems based on renewable energy sources can also be

explored. The flexibility and effectiveness of real-time power

factor correction systems can be further enhanced by

addressing these issues and expanding the scope of the study

Overall, implementing power factor correction helps stabilize

power systems, enhancing the efficiency of both the system and

connected equipment. The use of a microcontroller adds costeffectiveness

by enabling control over multiple parameters while

minimizing the need for additional hardware like timers, RAM,

ROM, and extra I/O ports. This project demonstrates a practical

and scalable solution for real-time power factor correction in

various settings